INTRODUCTION TO COMPUTER NETWORKS IST1104

GROUP1 PRESENTATION ASSIGNMENT1

SEMESTER ONE, YEAR ONE, 2026.

LECTURER: MR.MALE VINCENT

DATE: 02/09/2026

GROUP 1 MEMBERS:

QUESTION 1.

Discuss the seven layers of OSI model, specifying the responsibilities of each layer and the protocols that operate at each layer and what they do?

SOLUTION:

The OSI (Open Systems Interconnection) model is a conceptual framework developed by the International Organization for Standardization (ISO) to explain how data moves between devices over a network. It divides network communication into seven layers, with each layer having specific responsibilities and protocols.

| Layer | Name | Main Responsibility | Examples of Protocols |
| --- | --- | --- | --- |
| 7 | Application | Provides network services to applications | HTTP, HTTPS, FTP, SMTP, DNS, SSH |
| 6 | Presentation | Data formatting, encryption, compression | TLS/SSL, JPEG, PNG, ASCII, UTF-8 |
| 5 | Session | Establishes and manages communication sessions | NetBIOS, RPC, SIP |
| 4 | Transport | End-to-end delivery, reliability, flow control | TCP, UDP, SCTP |
| 3 | Network | Routing and logical addressing | IP, ICMP, IPsec, OSPF, BGP |
| 2 | Data Link | Frames, MAC addressing, error detection | Ethernet, Wi-Fi (802.11), PPP, ARP |
| 1 | Physical | Transmission of raw bits over a medium | Ethernet physical standards, USB, DSL, fiber |

- 1. Physical Layer

- The Physical Layer is the lowest layer of the OSI model. It deals with the actual transmission of raw binary bits (0s and 1s) through a physical medium.

- Responsibilities

  - Defines electrical, optical, or radio signals used to transmit data.

  - Specifies cables, connectors, frequencies, voltages, and signal timing.

  - Determines how individual bits are represented and transmitted.

  - Defines physical network topology and transmission media.

### Examples

  - Ethernet physical standards — transmit data over copper or fiber.

  - Wi-Fi radio standards (IEEE 802.11) — transmit bits using radio waves.

  - DSL — carries digital data over telephone lines.

  - Fiber-optic standards — transmit data using pulses of light.

  - USB — defines physical communication between devices.

Data unit: Bits

## 2. Data Link Layer

The Data Link Layer provides reliable communication between two devices that are directly connected on the same network segment.

### Responsibilities

  - Packages bits into frames.

  - Uses MAC addresses to identify devices on a local network.

  - Detects transmission errors, commonly using checksums such as CRC.

  - Controls access to the shared transmission medium.

  - Provides link-level flow control in some technologies.

### Examples of protocols/technologies

Ethernet (IEEE 802.3)
Defines how devices communicate on wired local-area networks, including Ethernet frames and MAC addressing.

Wi-Fi (IEEE 802.11)
Provides wireless LAN communication, including wireless frames and media-access mechanisms.

PPP (Point-to-Point Protocol)
Provides data-link communication over point-to-point connections.

ARP (Address Resolution Protocol)
Maps an IPv4 address to a device's MAC address on a local network. ARP is often associated with Layer 2, although its exact OSI classification is somewhat debated.

Data unit: Frame

## 3. Network Layer

The Network Layer is responsible for delivering packets between different networks. Its major functions are logical addressing and routing.

### Responsibilities

  - Provides logical addressing, such as IP addresses.

  - Determines the best route from source to destination.

  - Forwards packets between networks.

  - Handles packet fragmentation in protocols that support it.

  - Provides mechanisms for reporting certain network-level errors.

### Examples of protocols

IP (Internet Protocol)
Provides logical addressing and packet delivery between networks.

  - IPv4 uses 32-bit addresses.

  - IPv6 uses 128-bit addresses.

ICMP (Internet Control Message Protocol)
Carries network-control and error-reporting messages. For example, the ping utility uses ICMP Echo messages.

OSPF (Open Shortest Path First)
A routing protocol used within an autonomous system to calculate routes based on link-state information.

BGP (Border Gateway Protocol)
Exchanges routing information between autonomous systems and is fundamental to Internet routing.

IPsec
Provides security services such as authentication and encryption for IP communications.

Data unit: Packet

## 4. Transport Layer

The Transport Layer provides communication between applications running on different devices. It is responsible for end-to-end delivery.

### Responsibilities

  - Divides application data into smaller units when necessary.

  - Reassembles data at the destination.

  - Provides reliability when required.

  - Performs flow control.

  - Provides error recovery.

  - Uses port numbers to deliver data to the appropriate application.

  - Can provide congestion-control mechanisms.

### Major protocols

TCP (Transmission Control Protocol)
TCP provides a reliable, connection-oriented byte stream.

It uses:

  - Sequence numbers

  - Acknowledgements

  - Retransmissions

  - Flow control

  - Congestion control

For example, when downloading a file, TCP helps ensure that missing data is retransmitted and that the data arrives in the correct order.

UDP (User Datagram Protocol)
UDP is connectionless and has much less overhead than TCP. It does not guarantee delivery, ordering, or retransmission.

It is useful when low overhead and speed are more important than guaranteed delivery, such as DNS queries, streaming, and many real-time applications.

SCTP (Stream Control Transmission Protocol)
Provides reliable, message-oriented transport with features such as multistreaming and multihoming.

Data unit: Segment (TCP) / Datagram (UDP)

## 5. Session Layer

The Session Layer manages sessions or ongoing conversations between applications.

### Responsibilities

  - Establishes communication sessions.

  - Maintains and manages sessions.

  - Terminates sessions.

  - Provides synchronization points/checkpoints.

  - Can help resume communication after an interruption.

For example, a session mechanism can establish a logical conversation between two applications and manage when that conversation starts and ends.

### Examples

NetBIOS
Provides session services for applications communicating over a network, historically common in Windows networking.

RPC (Remote Procedure Call)
Allows an application to invoke a procedure on another computer as though it were a local procedure.

SIP (Session Initiation Protocol)
Establishes, modifies, and terminates multimedia communication sessions such as VoIP calls.

Important note: In modern TCP/IP networks, the functions of the OSI Session Layer are often implemented as part of the application layer or by application frameworks rather than as a distinct protocol layer.

Data unit: Data

## 6. Presentation Layer

The Presentation Layer ensures that data is represented in a format that the receiving application can understand.

Think of it as the translator and formatter of the OSI model.

### Responsibilities

  - Data translation and format conversion.

  - Character encoding.

  - Data compression and decompression.

  - Data encryption and decryption.

  - Serialization and representation of data.

### Examples

TLS (Transport Layer Security)
Provides encryption, authentication, and integrity protection for network communications. Although commonly associated conceptually with the Presentation/Session functions of OSI, TLS actually operates across the boundary between application and transport layers in practical TCP/IP networking.

JPEG/PNG
Define standardized representations for images.

ASCII/UTF-8
Define ways of representing characters as bytes.

Data serialization formats such as JSON and XML can also perform presentation-like functions, although they are generally considered application-layer technologies in the TCP/IP model.

Data unit: Data

## 7. Application Layer

The Application Layer is the highest OSI layer. It provides network services directly to applications and users.

It does not mean the application itself; rather, it defines the protocols applications use to communicate over a network.

### Responsibilities

  - Provides network services to applications.

  - Enables resource sharing.

  - Provides services such as web access, email, file transfer, and name resolution.

  - Defines application-level requests and responses.

### Examples of protocols

HTTP (Hypertext Transfer Protocol)
Used to transfer web resources between clients and servers.

HTTPS
HTTP carried over a secure connection, typically using TLS.

FTP (File Transfer Protocol)
Used for transferring files between computers.

SMTP (Simple Mail Transfer Protocol)
Used primarily to send and relay email.

DNS (Domain Name System)
Translates domain names such as example.com into IP addresses and performs other name-related functions.

SSH (Secure Shell)
Provides secure remote login and command execution.

DHCP (Dynamic Host Configuration Protocol)
Automatically provides hosts with network configuration such as IP addresses, subnet information, and default gateways.

Data unit: Data/message

# How the Seven Layers Work Together

When you send a message over a network, the data moves down the OSI layers on the sending computer:

Application → Presentation → Session → Transport → Network → Data Link → Physical

Each layer adds information needed by that layer. This process is called encapsulation.

At the receiving computer, the process is reversed:

Physical → Data Link → Network → Transport → Session → Presentation → Application

This is called decapsulation.

For example, when you visit a website:

  - Application: HTTP creates a web request.

  - Presentation: Data may be encrypted using TLS.

  - Session: The communication session is managed.

  - Transport: TCP provides reliable delivery and uses port 443 for HTTPS traffic.

  - Network: IP adds source and destination IP addresses and routers forward the packet.

  - Data Link: Ethernet or Wi-Fi creates a frame containing MAC addresses.

  - Physical: The frame is converted into electrical, optical, or radio signals and transmitted.

### Easy way to remember the layers

From Layer 7 down to Layer 1:

All People Seem To Need Data Processing

  - A — Application

  - P — Presentation

  - S — Session

  - T — Transport

  - N — Network

  - D — Data Link

  - P — Physical

### Key point

The OSI model is primarily a conceptual framework. Real-world Internet protocols do not always fit perfectly into one OSI layer. For example, TLS, ARP, and DNS are commonly discussed using OSI terminology but don't map cleanly to a single layer in every implementation. The model is nevertheless extremely useful for understanding networks and troubleshooting problems layer by layer.

QUESTION2.

Discuss the TCP/IP model, specifying its layers and the responsibilities of each layer and protocols that operate on each layer and what the protocols do?

SOLUTION:

# TCP/IP Model

The TCP/IP model (Transmission Control Protocol/Internet Protocol model) is the networking framework used as the foundation of the modern Internet. It describes how data is communicated between devices by dividing networking functions into four layers.

Unlike the seven-layer OSI model, the traditional TCP/IP model has four layers:

  - Application Layer

  - Transport Layer

  - Internet Layer

  - Network Access Layer

## 1. Application Layer

The Application Layer is the top layer of the TCP/IP model. It provides network services directly to applications and combines the functions of the Application, Presentation, and Session layers of the OSI model.

### Responsibilities

  - Provides services that network applications use to communicate.

  - Defines the format and meaning of application-level messages.

  - Handles data representation and, in some cases, encryption and compression.

  - Establishes and manages application-level communication.

  - Provides services such as web browsing, email, file transfer, and name resolution.

### Major protocols

| Protocol | Full name | What it does |
| --- | --- | --- |
| HTTP | Hypertext Transfer Protocol | Transfers web pages and other web resources between clients and servers. |
| HTTPS | HTTP Secure | Provides HTTP communication protected by TLS encryption and authentication. |
| FTP | File Transfer Protocol | Transfers files between computers. |
| SMTP | Simple Mail Transfer Protocol | Sends and relays email messages. |
| POP3 | Post Office Protocol version 3 | Downloads email messages from a mail server to a client. |
| IMAP | Internet Message Access Protocol | Allows clients to access and manage email stored on a mail server. |
| DNS | Domain Name System | Converts domain names such as example.com into IP addresses and performs other DNS lookups. |
| DHCP | Dynamic Host Configuration Protocol | Automatically provides devices with IP configuration such as an IP address, subnet information, and gateway. |
| SSH | Secure Shell | Provides secure remote login and command execution. |
| Telnet | Teletype Network | Provides remote terminal access, but without the encryption provided by SSH. |
| SNMP | Simple Network Management Protocol | Allows network devices to be monitored and managed. |

### Example

When you type a website address into a browser, HTTP/HTTPS operates at the Application Layer to define the request and response between your browser and the web server.

Data unit: Message/Data

# 2. Transport Layer

The Transport Layer provides end-to-end communication between applications running on different devices.

It is responsible for determining how application data is delivered from one process to another.

### Responsibilities

  - Provides end-to-end communication.

  - Breaks application data into smaller units when necessary.

  - Reassembles data at the destination.

  - Uses port numbers to identify applications.

  - Provides reliable delivery when required.

  - Performs error detection and recovery.

  - Provides flow control.

  - TCP also provides congestion control.

  - Can provide either connection-oriented or connectionless communication.

### Major protocols

## TCP — Transmission Control Protocol

TCP is a connection-oriented and reliable transport protocol.

It provides:

  - Reliable delivery.

  - Correct ordering of data.

  - Error detection.

  - Retransmission of lost data.

  - Flow control.

  - Congestion control.

  - Connection establishment and termination.

TCP establishes a connection using a three-way handshake:

SYN → SYN-ACK → ACK

For example, HTTPS commonly uses TCP as its underlying transport.

## UDP — User Datagram Protocol

UDP is a connectionless transport protocol with low overhead.

It does not guarantee:

  - Delivery

  - Ordering

  - Retransmission

UDP is useful when speed and low overhead are more important than guaranteed delivery.

Examples include:

  - DNS queries

  - Real-time voice/video

  - Online gaming

  - Streaming applications

## SCTP — Stream Control Transmission Protocol

SCTP is a reliable, message-oriented transport protocol. It supports features such as multistreaming and multihoming and is used in specialized networking applications.

Data unit: Segment (TCP) / Datagram (UDP)

# 3. Internet Layer

The Internet Layer is responsible primarily for logical addressing and routing packets between different networks.

It corresponds broadly to the Network Layer (Layer 3) of the OSI model.

### Responsibilities

  - Provides logical addressing.

  - Determines how packets travel between networks.

  - Routes packets from source to destination.

  - Forwards packets through routers.

  - Provides packet fragmentation/reassembly mechanisms where applicable.

  - Provides network-level error and diagnostic information through supporting protocols.

### Major protocols

## IP — Internet Protocol

IP is the most important protocol at this layer.

There are two major versions:

### IPv4

Uses 32-bit addresses, for example:

192.168.1.10

IPv4 provides addressing and routing of packets across interconnected networks.

### IPv6

Uses 128-bit addresses, for example:

2001:db8::1

IPv6 was developed partly to provide a vastly larger address space than IPv4.

## ICMP — Internet Control Message Protocol

ICMP is used to send error reports and diagnostic/control messages related to IP communication.

For example, the ping command commonly uses ICMP Echo Request and Echo Reply messages to test reachability.

## ICMPv6

ICMPv6 performs similar functions for IPv6 and also plays an important role in IPv6 network operation, including functions used by Neighbor Discovery.

## IGMP — Internet Group Management Protocol

IGMP is used by IPv4 hosts and routers to manage multicast group membership.

For example, it helps routers determine which local hosts want to receive multicast traffic.

## IPsec — Internet Protocol Security

IPsec provides security for IP communication, including:

  - Authentication

  - Integrity protection

  - Confidentiality/encryption

It is commonly used in VPNs and other secure IP communications.

## Routing protocols

Routing protocols help routers learn and exchange information about available paths.

Examples include:

  - OSPF — calculates routes within an autonomous system using link-state information.

  - RIP — uses a distance-vector approach based on hop count.

  - BGP — exchanges routing information between autonomous systems and is fundamental to Internet routing.

Data unit: Packet/Datagram

# 4. Network Access Layer

The Network Access Layer, also called the Link Layer or Network Interface Layer, is the lowest layer of the traditional TCP/IP model.

It combines functions corresponding roughly to the Data Link and Physical layers of the OSI model.

### Responsibilities

  - Provides communication over the local network.

  - Packages IP packets into frames.

  - Uses physical or hardware addresses such as MAC addresses.

  - Controls access to the transmission medium.

  - Detects errors in frames.

  - Defines how data is transmitted over the physical medium.

  - Handles physical signaling through copper, fiber, or radio.

### Major protocols and technologies

## Ethernet — IEEE 802.3

Ethernet is widely used for wired LANs.

It defines:

  - Ethernet frames.

  - MAC addressing.

  - Physical transmission standards.

  - Methods for communicating over wired networks.

For example, a computer connected to a switch using an Ethernet cable uses Ethernet at the Network Access Layer.

## Wi-Fi — IEEE 802.11

Wi-Fi provides wireless LAN communication.

It defines:

  - Wireless frames.

  - MAC addressing.

  - Wireless channel access.

  - Radio transmission mechanisms.

## PPP — Point-to-Point Protocol

PPP provides data-link communication over point-to-point connections. It has historically been used for connections such as dial-up and some WAN links.

## ARP — Address Resolution Protocol

ARP is used in IPv4 networks to discover the MAC address associated with a known IPv4 address on the local network.

For example:

"Who has 192.168.1.20?"

The device with that IP address can respond with its MAC address.

ARP is commonly associated with the TCP/IP Network Access/Link layer, although its exact placement in the OSI model is sometimes debated.

Data unit: Frame

# How the TCP/IP Layers Work Together

Suppose you open an HTTPS website.

The data travels through the TCP/IP layers as follows:

### 1. Application Layer

Your browser creates an HTTPS request.

Example:

"Please send me the webpage at example.com."

### 2. Transport Layer

TCP establishes a connection and provides reliable delivery.

The data is divided into TCP segments and assigned a destination port, typically 443 for HTTPS.

### 3. Internet Layer

IP adds source and destination IP addresses.

Routers use these addresses to determine where to forward the packet.

### 4. Network Access Layer

Ethernet or Wi-Fi puts the IP packet into a frame and transmits it across the local network.

The physical network then carries the frame as electrical, optical, or radio signals.

At the receiving computer, the process is reversed:

Network Access → Internet → Transport → Application

# TCP/IP Model vs. OSI Model

The two models are closely related but organized differently.

| TCP/IP Model | OSI Model | Main Function |
| --- | --- | --- |
| Application | Application | Network services for applications |
|  | Presentation | Data representation, encryption, compression |
|  | Session | Session management |
| Transport | Transport | End-to-end communication |
| Internet | Network | IP addressing and routing |
| Network Access | Data Link | Frames and MAC addressing |
|  | Physical | Signals and physical transmission |

Thus, the TCP/IP Application Layer combines OSI Layers 5–7, while the TCP/IP Network Access Layer combines OSI Layers 1–2.

# Summary

The TCP/IP model can be remembered as:

Application → Transport → Internet → Network Access

  - Application Layer: Provides services used by network applications. Examples: HTTP, HTTPS, DNS, FTP, SMTP, SSH.

  - Transport Layer: Provides end-to-end communication and uses ports. Examples: TCP, UDP, SCTP.

  - Internet Layer: Provides IP addressing and routing. Examples: IPv4, IPv6, ICMP, IGMP, IPsec, OSPF, BGP.

  - Network Access Layer: Handles local-network communication, framing, MAC addressing, and physical transmission. Examples: Ethernet, Wi-Fi, PPP, ARP.

The key idea is that each layer performs a specific networking function while cooperating with the layers above and below it, allowing data to travel from an application on one device to an application on another device across potentially many interconnected networks.

QUESTION3.

Compare and contrast the OSI model and the TCP/IP model (their similarities and differences and where they are applied)?

SOLUTION:

# Comparison Between the OSI and TCP/IP Models

The OSI (Open Systems Interconnection) model and the TCP/IP (Transmission Control Protocol/Internet Protocol) model are both reference frameworks used to understand how computers communicate over networks. They divide networking functions into layers, but they differ in their structure, purpose, and practical use.

## 1. Similarities

The two models have several important similarities:

  - Both use a layered architecture. Network communication is divided into separate layers, with each layer performing specific functions.

  - Both describe how data travels between devices. Data moves down the layers at the sender and up the layers at the receiver.

  - Both use encapsulation. Each layer adds information to data as it moves toward the network.

  - Both include equivalent transport and network/routing functions. TCP/IP's Transport and Internet layers correspond closely to OSI's Transport and Network layers.

  - Both promote modularity. Changes to one layer can be made without necessarily changing all other layers.

  - Both are useful for network troubleshooting and education. They help network engineers identify where a communication problem occurs.

  - Both provide a common vocabulary for discussing network communication and protocols.

# 2. Differences Between the OSI and TCP/IP Models

| Feature | OSI Model | TCP/IP Model |
| --- | --- | --- |
| Full name | Open Systems Interconnection | Transmission Control Protocol/Internet Protocol |
| Number of layers | 7 | Traditionally 4 |
| Developed by | ISO | U.S. Department of Defense/ARPA research community |
| Primary purpose | General reference model for networking | Practical framework for Internet/network protocols |
| Application functions | Application, Presentation, Session are separate | Combined into Application layer |
| Physical/Data Link functions | Separate Physical and Data Link layers | Combined into Network Access/Link layer in the traditional 4-layer model |
| Protocol dependence | Designed as a protocol-independent reference model | Developed around the TCP/IP protocol suite |
| Practical implementation | Mainly conceptual/educational | Widely implemented in real networks |
| Layer boundaries | More strictly defined | More flexible |
| Protocol development | Model was developed before many corresponding protocols | Protocols and architecture evolved together |
| Typical use | Teaching, design, troubleshooting, conceptual analysis | Internet and real-world network communication |

# 3. Difference in Number of Layers

The biggest structural difference is the number of layers.

### OSI — 7 Layers

From top to bottom:

  - Application

  - Presentation

  - Session

  - Transport

  - Network

  - Data Link

  - Physical

### TCP/IP — 4 Layers

From top to bottom:

  - Application

  - Transport

  - Internet

  - Network Access

The relationship can be represented as:

OSI MODEL                    TCP/IP MODEL

7. Application ───────┐

6. Presentation ────────┼──> Application

5. Session ─────────────┘

4. Transport ──────────────> Transport

3. Network ────────────────> Internet

2. Data Link ────────────┐

1. Physical ────────────┴──> Network Access

Therefore:

  - TCP/IP Application ≈ OSI Application + Presentation + Session

  - TCP/IP Transport ≈ OSI Transport

  - TCP/IP Internet ≈ OSI Network

  - TCP/IP Network Access ≈ OSI Data Link + Physical

# 4. Difference in Purpose

### OSI Model

The OSI model was designed primarily as a general networking reference model. It describes what networking functions should occur at each layer without being tied to a particular protocol suite.

It is particularly useful for:

  - Learning networking concepts.

  - Designing and analyzing networks.

  - Troubleshooting network problems.

  - Understanding where different protocols and technologies operate.

  - Providing a common framework for networking discussions.

For example, if a network cable is damaged, you can reason that the problem is at the Physical Layer. If routing is incorrect, you can investigate the Network Layer.

### TCP/IP Model

The TCP/IP model was developed around the actual protocols used for interconnected computer networks and ultimately became the foundation of the modern Internet.

It is used for:

  - Internet communication.

  - Local and wide-area networks.

  - Corporate networks.

  - Data centers.

  - Cloud networks.

  - Wireless networks.

  - Internet-connected applications and services.

# 5. Difference in Protocols

The OSI model is protocol-independent. It provides a framework into which protocols can be categorized.

The TCP/IP model, on the other hand, is closely associated with the TCP/IP protocol suite.

Examples include:

### OSI model

Protocols and technologies can be mapped to the appropriate OSI layer:

  - Application — HTTP, DNS, SMTP

  - Transport — TCP, UDP

  - Network — IP, ICMP, OSPF

  - Data Link — Ethernet, Wi-Fi

  - Physical — copper, fiber, radio transmission

### TCP/IP model

The model was built around protocols such as:

  - Application: HTTP, DNS, SMTP, FTP, SSH

  - Transport: TCP, UDP

  - Internet: IP, ICMP, IGMP

  - Network Access: Ethernet, Wi-Fi, PPP

# 6. Difference in Application

## Where the OSI Model is Applied

The OSI model is mainly applied as a reference and teaching framework rather than as a protocol suite that networks directly implement.

It is commonly used in:

  - Networking education.

  - Network certification training.

  - Network troubleshooting.

  - Protocol analysis.

  - Network architecture and design.

  - Explaining security controls and network devices.

For example, network engineers often describe a switch as primarily operating at Layer 2 and a router as primarily operating at Layer 3, using OSI terminology.

## Where the TCP/IP Model is Applied

TCP/IP is used extensively in actual network communication.

It forms the basis of:

  - The Internet.

  - Web communication.

  - Email.

  - File transfers.

  - Cloud computing.

  - Enterprise networks.

  - Wireless networking.

  - Virtual private networks.

  - Data-center networking.

  - IoT communication.

For example, when you access a website, protocols such as HTTPS, TCP, IP, and Ethernet/Wi-Fi work together according to the TCP/IP architecture.

# 7. Advantages of the OSI Model

The OSI model has several advantages:

  - Easy to understand: Seven clearly separated layers make networking concepts easier to learn.

  - Detailed separation of functions: Application, presentation, and session functions are distinguished.

  - Excellent for troubleshooting: Problems can be isolated to particular layers.

  - Protocol independent: It can be used to describe different networking technologies.

  - Provides standard terminology: Network professionals can communicate using concepts such as "Layer 2" and "Layer 3."

### Limitation

The main limitation is that it is not a direct representation of how the modern Internet protocol suite is implemented. Some protocols do not fit neatly into one OSI layer.

# 8. Advantages of the TCP/IP Model

The TCP/IP model has several advantages:

  - Practical: It is closely associated with protocols actually used on networks.

  - Scalable: It supports communication across networks of virtually any size.

  - Widely implemented: It is the foundation of Internet communication.

  - Interoperable: Different manufacturers' devices can communicate using standardized TCP/IP protocols.

  - Flexible: It supports many types of physical and data-link technologies.

### Limitation

Its four-layer structure provides less separation of functions than the OSI model. For example, presentation and session functions are incorporated into the Application layer.

# 9. Example: Accessing a Website

Suppose you type:

https://www.example.com

into a browser.

### Using the OSI model

  - Application: Browser creates an HTTP request.

  - Presentation: Data may be encrypted/represented using mechanisms such as TLS and appropriate data formats.

  - Session: Communication sessions are managed.

  - Transport: TCP provides reliable end-to-end communication.

  - Network: IP provides addressing and routing.

  - Data Link: Ethernet or Wi-Fi creates frames and handles local delivery.

  - Physical: Bits are transmitted through cable, fiber, or radio.

### Using the TCP/IP model

  - Application: HTTPS handles the web communication.

  - Transport: TCP provides reliable transport.

  - Internet: IP handles addressing and routing.

  - Network Access: Ethernet or Wi-Fi handles local transmission.

The same communication is being described; the models simply divide the functions differently.

# 10. Overall Comparison

| Aspect | OSI | TCP/IP |
| --- | --- | --- |
| Layers | 7 | 4 traditionally |
| Nature | Reference model | Protocol architecture/model |
| Main purpose | Explain and standardize networking concepts | Enable practical network communication |
| Use today | Education, troubleshooting, conceptual design | Actual Internet and network communication |
| Application/Presentation/Session | Separate | Combined |
| Data Link/Physical | Separate | Combined in traditional model |
| Protocol relationship | Protocol-independent | Closely tied to TCP/IP protocols |
| Internet use | Reference framework | Fundamental architecture |
| Troubleshooting | Very useful | Also useful, especially for TCP/IP networks |

## Conclusion

The OSI and TCP/IP models are similar because both use layers to organize network communication and explain how data moves between devices. However, they differ significantly in structure and practical application.

The OSI model has seven layers and is primarily a conceptual/reference model used for learning, analysis, design, and troubleshooting. The TCP/IP model traditionally has four layers and is a practical networking architecture built around protocols such as TCP, IP, HTTP, and DNS.

In simple terms:

OSI explains networking in greater detail, while TCP/IP describes the architecture that is actually used to communicate across the Internet and most modern IP networks.